Attribution analysis using the Budyko method

Based on the Budyko hypothesis, the analytical water-energy balance equation is derived at the mean annual time scale, which is expressed as:

E=P×PETPn+PETn1n#1

where E is the mean annual actual evapotranspiration, P is the mean annual precipitation, PET is the mean annual potential evapotranspiration, which is calculated following the guidelines provided by the Food and Agriculture Organization, and the parameter n represents the basin landscape characteristics, which include properties of soil, topography, and vegetation.

Assuming negligible changes in terrestrial water storage and combining the long-term basin water balance with equation (1), we obtain:

Qn=P- P×PETPn+PETn1n#2

where Qn is the mean annual naturalized streamflow.

Assuming P, PET, and n are independent variables and introducing the concept of

elasticity, we obtain the following:

dQnQn= εPdPP+εPETdPETPET+εndnn#3

εP= dQnQndPP= 1-PETPn1+PETPn1n+11-PETPn1+PETPn1n#4

εPET= dQnQndPETPET=11+PETPn11-1+PETPnPETPn1n#5

εn=dQnQndnn= 11+PPETn1n-1PnlnP+PETnlnPETPn+PETn-lnPn+PETnn#6

where εP, εPET, and εn represent P, PET, and n elasticity of streamflow, respectively. Based on equation (3), Budyko-simulated changes in naturalized streamflow (∆Qn) can be attributed to the CCV-induced change (∆Qn,CVV) and the LUCC-induced change (∆Qn,LUCC):

∆Qn=εPQnP∆P+εPETQnPET∆PET+εnQnn∆n#7

where Δ represents the changes from the pre-1986 period to 1986 onwards. Based on equation (7), the contributions of CCV, LUCC, and WADR to changes in observed streamflow (∆Qo), as calculated using the Budyko method, can be expressed as:

CCCV= ∆Qn,CVV∆Qo×100%#8

CLUCC= ∆Qn,LUCC∆Qo×100%#9

CWADR= ∆Qo-∆Qn∆Qo×100%#10

The Budyko attribution framework is applied through the following steps:

- Step 1: input the 1960-2016 annual mean P, PET, and Qn into equation (2) to inversely calculate the parameter n for each station.

- Step 2: input the 1960-2016 annual mean P, PET, and the parameter n (obtained in Step 1) into equations (4-6) to calculate εP, εPET, and εn.

- Step 3: input the annual mean P, PET, and Qn for the pre-1986 period and from 1986 onwards into equation (2) to inversely calculate the parameter n for each station in both periods.

- Step 4: input ΔP, ΔPET, and Δn from the pre-1986 period to 1986 onwards, together with Qn, P, PET, n, εP, εPET, and εn in Steps 1 and 2, into the equation (7) to calculate ∆Qn, ∆Qn,CVV, and ∆Qn,LUCC.

- Step 5: input changes in observed (ΔQo) and naturalized (ΔQn) streamflow from the pre1986 period to 1986 onwards, together with ∆Qn,CVV and ∆Qn,LUCC obtained in Step 4, into equations (8-10) to calculate contribution rates.

- Step 6: use ∆Qn obtained in Step 4 and ΔQn to plot fig. S5A. Since Qn is used in Step 1 to inversely calculate the parameter n in the Budyko model (serving as model calibration), the Budyko-simulated naturalized streamflow exhibits perfect agreement and correlation. This confirms that the Budyko framework effectively quantifies the relationships between P, PET, and Qn.

ISIMIP3a,

The observation-based forcings used in these models are specified as “obsclim” and “histsoc.”

1) “obsclim” refers to observation-based climate-related forcings, consisting of standard atmospheric forcings. ISIMIP3a uses four atmospheric forcings—GSWP3-W5E5, 20CRv3, 20CRv3-W5E5, and 20CRv3-ERA5—each of which is used separately to drive the GHMs.

2) “histsoc” represents varying direct human influences during the historical period, including LUCC, variable human water abstraction, and simplified reservoir regulation.

To isolate and identify the contributions of different drivers to streamflow changes, ISIMIP3a transforms observation-based forcings and uses these transformations to rerun the GHMs. The transformations used in this study are “counterclim” and “1901soc”.

3) “counterclim” refers to a hypothetical counterfactual climate condition in the absence of observed climate change. This condition is derived by detrending four atmospheric forcings using ATTRICI version 1.1, which removes global mean temperature–related shifts in each grid.

4) “1901soc” represents direct human influences that remain constant over time, maintaining land use, human water abstraction, and reservoir conditions identical to those in 1901.

On the basis of the information outlined above, Qo is reconstructed using GHMs forced by “obsclim + histsoc” (denoted as Q′o), and Qn is reconstructed using GHMs forced by “obsclim + 1901soc” (denoted as Q′n). In addition, GHMs forced by “counterclim + 1901soc” (denoted as Q′cn) are used to distinguish the influence of climate change from climate variability. According to the GHMs participating in ISIMIP3a, as listed in data S2, we downloaded Q′o, Q′n, and Q′cn from nine ISIMIP3a outputs.

the contributions of CCV, LUCC, and WADR to the change in Qo denoted as CCCV,CLUCC, and CWADR, can be calculated as

CCCV= ∆Qn,∆Qo×100

CLUCC=∆Qn-∆Qn'∆Qo×100

CWADR=∆Qo-∆Qn∆Qo×100

CCCV can be further divided into the contributions of ACC and NCV, namely CACC and CNCV, as follows

CACC= ∆Qn'-∆Qcn'∆Qo×100

CACC= ∆Qcn'∆Qo×100

ISIMIP2a, NOSOC, VARSOC experiment

data sources:

Observed streamflow data: Global Runoff Data Center (GRDC, https://portal.grdc.bafg.de/applications/public.html?publicuser=PublicUser#dataDownload/Home)

Water consumption data of different sectors-irrigation, livestock, electricity generation, domestic, mining and manufacturing from 1970-2010 (https://doi.org/10.5281/zenodo.1209296)

Future climate projections: https://www.isimip.org/

References:

Rosa L, Sangiorgio M. Global water gaps under future warming levels[J]. Nature Communications, 2025, 16(1): 1192.

Deng Q, Sharretts T, Ali T, et al. Deepening water scarcity in breadbasket nations[J]. Nature Communications, 2025, 16(1): 1110.

Wang K, Liu X, Cui P, et al. China’s nationwide streamflow decline driven by landscape changes and human interventions[J]. Science Advances, 2025, 11(32): eadu8032.

Liu X, Liu W, Yang H, et al. Multimodel assessments of human and climate impacts on mean annual streamflow in China[J]. Hydrology and Earth System Sciences, 2019, 23(3): 1245-1261.

Huang Z, Yuan X, Liu X. The key drivers for the changes in global water scarcity: Water withdrawal versus water availability[J]. Journal of Hydrology, 2021, 601: 126658.

He S, Chen K, Liu Z, et al. Exploring the impacts of climate change and human activities on future runoff variations at the seasonal scale[J]. Journal of Hydrology, 2023, 619: 129382.

Liu W, Fu Z, van Vliet M T H, et al. Global overlooked multidimensional water scarcity[J]. Proceedings of the National Academy of Sciences, 2025, 122(26): e2413541122.